Topic:- SMART IRRIGATION SYSTEM

Team Members and Roll No. :-

- Prithvi Raj Gupta	 (25/SET/BCML/126)

- Krish Aggarwal 	 (25/SET/BCML/63)

- Didhi Samiksha	 (25/SET/BCML/117)

- Satyam Jha  	 	(25/SET/BCML/83)

Role of Each Participant:-

- Prithvi Raj Gupta:

- Model assembly

- Writeup

- Krish Aggarwal:

- Model assembly

- Team leader

- Didhi Samiksha:

- Model assembly

- Writeup

- Satyam Jha:

- Model assembly

- Making connections

Professor Name (PHYSICS):- Dr. Sandeep Srivastava Sir

Aim:-

To make Smart Irrigation System using Arduino Uno.

Components Required: -

- Arduino Uno R3 (Type-C)

- Soil Moisture Sensor (Digital Output)

- Relay Module (5V)

- DC Motor/Water Pump (5V/12V)

- Jumper wires

- Breadboard

- External Power Supply (for motor if needed)

- Type C cable

- Hot glue gun

- Cardboard

- Plant pot

- Screw driver

Reference: https://share.google/images/UWlTNxsdRr23lnXXg

Reference:https://share.google/images/UvmzTiFM1B2YjuM7h

Theory: -

A Smart Irrigation System is an automated watering system that uses sensors and microcontrollers to supply water to plants only when required. Its main principle is soil-moisture-based irrigation, which helps in water conservation and efficient crop growth.

Core Theoretical Concepts: -

- Soil Moisture Sensing

- A soil moisture sensor (capacitive or resistive type) measures the water content of the soil.

- It provides an analog voltage to the Arduino Uno that corresponds to moisture levels:

- Dry soil → High analog value

- Wet soil → Low analog value

- Microcontroller-Based Decision Making

- The Arduino Uno reads the moisture sensor’s analog value through its ADC (Analog-to-Digital Converter).

- Based on a predefined threshold moisture level, the Arduino decides whether irrigation is needed.

- This forms a closed-loop control system.

- Water Pump Control

- A relay module acts as a switch to control a water pump (DC pump/submersible pump).

- When soil moisture falls below threshold → Arduino activates relay → pump ON

- When soil moisture rises above threshold → pump OFF

- Automation + Power Efficiency

- The system eliminates manual watering.

- Prevents over-irrigation and under-irrigation.

- Saves water by supplying it only when necessary.

- Optional Add-Ons (theoretical features)

- Temperature/humidity sensor (DHT11/22)

- Real-time clock (RTC) for time-based control

- GSM module for SMS alerts

- Wi-Fi module (ESP8266) for IoT monitoring

- OLED/LCD display for real-time moisture status

Functions of Materials:-

Pin Connections:

- Soil Moisture Sensor:

- VCC -> 5V

- GND -> GND

- D0 -> Pin 7 (Digital Output)

Reference: https://share.google/images/7FMNFSB3X6RInMq8A

- Relay Module:

- VCC -> 5V

- GND -> GND

- IN -> Pin 6 (Control Signal)

Reference:https://share.google/images/YY593TAGZo3HUEM7s

- DC Motor/Pump:

- Connect to Relay COM and NO terminals

- Motor power supply connected through relay

Reference: https://share.google/images/juLShkOtYPqVpqxSV

Construction: -

- Connect 5V to positive rail and GND to negative rail.

- Connect sensor VCC to positive rail and GND to negative rail.

- Single pin D7.

- CONNECT RELAY VCC 5V , GND to GND , single pin-D6.

- Connect jumper wires to mini pump black wire and connect the red wire of mini pump with the ND of single relay.

- Then take another jumper wire and connect it with COM of single relay.

- Now connect the jumper wires attached to single relay, black with negative, red with positive of breadboard.

- Put the code given below in tinkercad.

- Connect the whole model with computer.

Reference: https://youtu.be/bJIriNF0zcA?si=Pzr6Gw3DD3gJOM4

Block Diagram: -

Soil Moisture Sensor → Arduino Uno → Relay Module → Water Pump → Plant/Field

Feedback: -

Soil moisture updates continuously and regulates the pump.

Working Of System: -

- Soil moisture sensor is placed in the soil.

- Sensor sends continuous moisture data to Arduino.

- Arduino compares the reading with a preset threshold.

- If soil is dry:

→Arduino sends HIGH signal to relay

→Relay switches ON the pump

- Watering continues until moisture reaches the threshold.

- Then Arduino switches OFF the pump.

- Process keeps repeating automatically.

Pin Definitions (Code): -

const int soilMoisturePin = 7; // Soil moisture sensor digital pin

const int relayPin = 6; // Relay control pin

Variables

int soilMoistureState = 0;// Soil moisture digital reading

bool pumpState = false; // Current pump state

void setup() {

// Initialize Serial Monitor

Serial.begin(9600);

Serial.println("Smart Irrigation System Started");

Serial.println("================================");

Serial.println("Pin 6: Relay (Motor Control)");

Serial.println("Pin 7: Soil Moisture Sensor");

Serial.println("================================");

// Set pin modes

pinMode(soilMoisturePin, INPUT);   // Soil sensor as input

pinMode(relayPin, OUTPUT);          // Relay as output

// Initial state - pump OFF

digitalWrite(relayPin, LOW);

pumpState = false;

delay(2000); // Wait for sensor to stabilize

}

void loop() {

// Read soil moisture sensor (Digital)

// Most sensors: LOW = Wet soil, HIGH = Dry soil

soilMoistureState = digitalRead(soilMoisturePin);

// Display readings on Serial Monitor

Serial.print("Soil Moisture Sensor: ");

if (soilMoistureState == HIGH) {

// Soil is DRY - Turn pump ON

Serial.print("DRY (HIGH) | ");

digitalWrite(relayPin, HIGH);

pumpState = true;

Serial.println("Motor/Pump: ON - Watering...");

} else {

// Soil is WET - Turn pump OFF

Serial.print("WET (LOW) | ");

digitalWrite(relayPin, LOW);

pumpState = false;

Serial.println("Motor/Pump: OFF - Sufficient moisture");

}

delay(2000); // Check every 2 seconds

}

HOW IT WORKS: -

- Digital Soil Moisture Sensor:

- Output HIGH (1) when soil is DRY

- Output LOW (0) when soil is WET

- Relay Module:

- HIGH signal activates relay (motor ON)

- LOW signal deactivates relay (motor OFF)

- System Logic:

- Sensor detects dry soil → Sends HIGH to Pin 7

- Arduino sends HIGH to Pin 6 → Relay activates

- Motor/Pump turns ON → Waters the plant

- Soil becomes wet → Sensor sends LOW to Pin 7

- Arduino sends LOW to Pin 6 → Relay deactivates

- Motor/Pump turns OFF → Watering stops

IMPORTANT NOTES: -

- Pin 6: Controls the relay module (and thus the motor)

- Pin 7: Reads soil moisture sensor status

- Adjust sensor potentiometer to set sensitivity

- Use external power for motor if it draws >500mA

- Make sure relay can handle your motor's voltage/current

CALIBRATION: -

- Turn the potentiometer on the soil moisture sensor

- Clockwise = Less sensitive (triggers at drier soil)

- Counter-clockwise = More sensitive (triggers at wetter soil)

- Test by inserting sensor in dry/wet soil and checking LED on sensor

TROUBLESHOOTING: -

- If motor doesn't run: Check external power supply connection

- If motor runs continuously: Adjust sensor potentiometer

- If readings inverted: Some sensors have inverted logic

- Open Serial Monitor (9600 baud) to see real-time status

Application: -

- Agriculture

- Automatic crop irrigation in farms.

- Reduces labor and water usage (up to 40%).

- Greenhouses

- Maintains controlled moisture levels for delicate plants.

- Home Gardening

- Useful for pots, rooftop gardens, and small nurseries.

- Smart Cities / IoT Projects

- Lawn irrigation in parks using IoT control.

- Integration with weather-based watering systems.

- Precision Farming

- Used in modern agriculture to monitor soil health and optimize watering cycles.

Reference:https://youtu.be/bJIriNF0zcA?si=Pzr6Gw3DD3gJOM4_

Advantages: -

- Saves water and electricity.

- Reduces human effort.

- Prevents over-irrigation (root rot) and under-irrigation (drying).

- Low cost and easy to implement.

- Highly customizable for different crops/climates.

Conclusion: -

A Smart Irrigation System using Arduino Uno is an efficient microcontroller-based solution that automates the watering process, improves water usage efficiency, and supports sustainable agriculture. With simple sensors and relays, the system becomes highly reliable, cost-effective, and suitable for farms, gardens, and IoT-based smart city applications.

For the construction of this model the refrences are :-

https://youtu.be/bJIriNF0zcA?si=Pzr6Gw3DD3gJOM4_